VSP3 → XFOIL → FLOWUnsteady Pipeline & Propeller Validation

# 1. Pipeline Overview

Input files

| File | Provides | Role |
| --- | --- | --- |
| .bem | Chord, twist, sweep, height, R, Rhub, blade count B | Source of truth for blade planform — written directly into the final Julia script |
| .vsp3 | Per-station airfoil geometry (Camber/CamberLoc/ThickChord, or raw UpperPnts/LowerPnts) | Source of airfoil shape only; used to generate XFOIL polars |

Processing sequence

- Parse .bem → chord/twist/sweep/height tables; duplicate hub row at r/R = 0 (required by FLOWVLM).

- Parse .vsp3 station blocks using a depth-counted XML scan (not a naive regex — <XSec> tags nest, and a lazy match silently drops or mis-splits stations).

- Classify each station: NACA Four-Series (Camber/CamberLoc/ThickChord present) vs. Custom imported (UpperPnts/LowerPnts present) vs. unsupported (skipped, logged).

- Pre-flight checks: consistency check (.bem vs .vsp3 tip radius/blade count) and geometry sanity checks (thick roots, thin+high-camber tips, large spanwise jumps) — informational, non-blocking.

- Generate contour coordinates per station, run each through XFOIL, assemble the final FLOWUnsteady .jl script referencing every contour + polar file.

NACA Four-Series stations

- Exact closed-form NACA 4-digit equations (cosine-spaced x, thickness envelope, cambered mean line) generate 100+ (x, y) points directly from Camber/CamberLoc/ThickChord — no lookup tables.

Custom (imported) airfoil stations

- Raw UpperPnts/LowerPnts arrays are extracted from the .vsp3 XML (stored as X, Y, Z triplets; Z is always 0 and discarded).

- Upper surface is reversed and stitched to the lower surface into continuous Selig order (TE → LE → TE); a duplicate seam point (an OpenVSP export artifact) is detected and removed; both trailing-edge endpoints are pinned to x = 1.0.

- Bypasses the NACA formula entirely — the imported shape is used exactly as designed in OpenVSP.

Security / validation checks

| Check | Purpose |
| --- | --- |
| Point-count validation | Custom contours below ~20 pts are rejected as unusable; 150–250 pts is the ideal band; excess (>400) is flagged as slow but not fatal |
| .bem ↔ .vsp3 consistency | Warns (does not silently correct) if tip radius or blade count disagree — catches unit/model mismatches early |
| Geometry sanity (pre-flight) | Flags thick roots, thin+high-camber tips, and large spanwise jumps before any XFOIL time is spent |
| Polar file validation | Every referenced polar is hard-checked to exist and contain data before the .jl script is finalized |
| No unit auto-correction | Dimensional inputs (e.g. inches vs. metres) are never guessed — flagged only, to avoid silently propagating an error into the physics |

XFOIL handoff and output

- Each station's .dat contour + estimated local Reynolds number is passed to XFOIL via a scripted command sequence (paneling, viscous OPER, AoA sweep).

- A fallback chain retries progressively narrower AoA sweeps if the full sweep fails to converge; a station that never converges gets its polar (never its geometry) substituted from the nearest converged station.

- Final output: a self-contained FLOWUnsteady .jl script, per-station .dat contours, and 7-column XFOIL polar CSVs — ready to run a full rotor simulation.

# 2. Propeller Aerodynamic Performance Validation

A 3-bladed, D = 0.3 m propeller was evaluated at J = 0.4 (9200 RPM, V∞ = 18.4 m/s) using two independent solvers — VSPAERO (3-D panel method, thick-set geometry) and FLOWUnsteady (viscous vortex particle method, VPM) — to cross-validate the computational setup and blade geometry.

Setup notes

- Simulation reached periodic steady state after ~2 rotor revolutions (720°); comparison data extracted at 1160° wake age so starting vortices had fully convected downstream.

- VSPAERO loads (computed on one blade) were multiplied by B = 3 for a full-rotor comparison.

Results

| Metric | VSPAERO | FLOWUnsteady | Difference |
| --- | --- | --- | --- |
| Thrust coeff. Cᴛ | 0.1960 | 0.1867 | ~5.0% |
| Torque coeff. C_Q | 0.0213 | 0.0203 | ~4.9% |
| Efficiency η | 58.56% | 58.50% | <0.1% |

Interpretation

- Efficiency agrees almost exactly because the ~5% over-prediction in VSPAERO scales proportionally across both thrust and torque — a strong internal-consistency signal, not a coincidence.

- The ~5% absolute force gap is attributed to wake modeling: FLOWUnsteady's free-vortex particle wake captures slipstream contraction, raising local induced velocity and slightly lowering effective angle of attack — hence lower predicted thrust.

- FLOWUnsteady also resolves full viscous interaction and 3-D rotational stall delay directly, versus VSPAERO's empirical skin-friction buildup.

- Both solvers correctly account for blade volume displacement, including thick root sections (t/c up to 20%).

Conclusion

The close agreement between a 3-D panel method and a high-fidelity viscous vortex particle method — particularly in propulsive efficiency — establishes high confidence in the evaluated propeller performance metrics at J = 0.4.